# Design Criteria

Updated November 9, 2023

## Adopted Codes

- 2018 International Building Code (IBC)

- 2018 International Fire Code (IFC)

- 2018 International Swimming Pool and Spa Code (ISPSC)

- 2018 International Existing Building Code (IEBC)

- 2018 International Residential Code (IRC)

- 2018 International Energy Conservation Code (IECC)

- 2018 International Mechanical Code (IMC)

- 2018 International Fuel & Gas Code (IFGC)

- 2017 Idaho State Plumbing Code (ISPC)

- 2023 National Electrical Code (NEC) – HB337 (IC 54-1001) effective 7/1/2023

## General Design Guidelines

If the information is not found below, refer to the adopted codes stated above for proper design methods to meet minimum compliance.

### Snow Load

- Ground snow loads = 20 psf and are also determined by section 7, of ASCE 7; however, the design roof load shall not be less than a uniform snow load of 25 psf.

- Roof Ice Barriers are not required

### Floor Loads

- Live load 40 psf

- Dead load 20 psf

- Total 60 psf or ASCE 7

- See Table 4-1 for conditions that would require a greater load

### Seismic Design

- C

### Lateral Loads

- ASCE 7 and adopted IBC

- Geotechnical report required

### Wind

- 100 mph risk occupancy II or less

- 115 mph for risk occupancy III, or greater

- Exposure category per IBC Section 1609.4

### Soil Properties

- Geotechnical report required for structures in seismic areas and for basements

### Frost line

- Minimum 24 inches deep footings

### Weathering

- Severe

### Exposure

- B

### Termite

- Slight to moderate

### Decay

- Slight to none

### Winter Design Temperature

- 10 degrees (f)

- Annual Mean Temperature - 51.1

### Floodplain Ordinance

- Refer to Title 10, Chapter 6, Meridian City Code, Flood Damage Prevention

### Climate Zone

- 5B

### Air Freezing Index

- 294 (National Climatic Data Center)